# WiFi Training Program – CAPWAP & AP Architecture Assignment

# 1. Split MAC Architecture

Split MAC architecture divides MAC functions between Access Point (AP) and Wireless LAN Controller (WLC).

AP handles time-critical tasks such as frame transmission, ACKs, encryption, and beaconing. WLC handles centralized functions like authentication, policy enforcement, roaming, and QoS.

This improves performance by reducing AP load, enabling centralized management, better scalability, and faster roaming.

# 2. CAPWAP and Flow

CAPWAP (Control and Provisioning of Wireless Access Points) connects APs to WLC using UDP ports 5246 (control) and 5247 (data).

Flow:
1. Discovery – AP finds WLC
2. Join – AP authenticates with WLC
3. Configuration – WLC pushes config
4. Data Transfer – Client traffic flows

# 3. CAPWAP OSI Layer and Tunnels

CAPWAP operates at Application Layer (Layer 7) using UDP.

Control Tunnel (UDP 5246): Used for configuration and management
Data Tunnel (UDP 5247): Used for client data transmission

# 4. Lightweight AP vs Cloud-based AP

Lightweight AP uses on-prem WLC, while Cloud-based AP uses cloud controller.

Lightweight: Local control, limited scalability
Cloud: Internet-based, highly scalable and flexible

# 5. CAPWAP Tunnel Maintenance

Maintained using keepalive messages between AP and WLC. If heartbeat fails, tunnel is considered down.

# 6. Sniffer Mode vs Monitor Mode

Sniffer Mode: Packet capture for troubleshooting
Monitor Mode: Security scanning and rogue AP detection

# 7. Best AP Mode for WAN WLC

FlexConnect mode is best. It allows local switching of traffic while control goes to WLC, reducing WAN dependency.

# 8. Challenges with Autonomous APs

No centralized management, difficult updates, poor scalability, inconsistent policies, and roaming issues.

# 9. WLC Failure Impact

In local mode, AP depends on WLC. If WLC fails, clients disconnect and new connections are not possible.